CHAPTER – 3 - STRENGTH CALCULATION AND DIMENSIONING OF JOINTS

RIVETED JOINTS

3.1.1 Introduction

A rivet is a short cylindrical bar with a head integral to it. The cylindrical portion of the rivet is called shank or body and lower portion of shank is known as tail. The rivets are used to make permanent fastening between the plates such as in structural work, ship building, bridges, tanks and boiler shells. The riveted joints are widely used for joining light metals .The fastenings (i.e. joints) may be classified into the following two groups:

1. Permanent fastenings, and  2. Temporary or detachable fastenings.

The permanent fastenings are those fastenings which can not be disassembled without destroying the connecting components. The examples of permanent fastenings in order of strength are soldered, welded and riveted joints.

The temporary or detachable fastenings are those fastenings which can be disassembled without destroying the connecting components. The examples of temporary fastenings are screwed, keys, cotters, pins and splined joints.

3.1.2 Methods of Riveting

The function of rivets in a joint is to make a connection that has strength and tightness. The strength is necessary to prevent failure of the joint. The tightness is necessary in order to contribute to strength and to prevent leakage as in a boiler or in a ship hull. When two plates are to be fastened together by a rivet as shown in Fig. 9.2 (a), the holes in the plates are punched and reamed or drilled. Punching is the cheapest method and is used for relatively thin plates and in structural work. Since punching injures the material around the hole, therefore drilling is used in most  pressure-vessel work. In structural and pressure vessel riveting, the diameter of the rivet hole is usually 1.5 mm larger than the nominal diameter of the rivet.

The riveting may be done by hand or by a riveting machine. In hand riveting, the original rivet head is backed up by a hammer or heavy bar and then the die or set, as shown in Fig. 9.2 (a), is placed against the end to be headed and the blows are applied by a hammer. This causes the shank to expand thus filling the hole and the tail is converted into a point as shown in Fig. 9.2 (b). As the rivet cools, it tends to contract. The lateral contraction will be slight, but there will be a longitudinal tension introduced in the rivet which holds the plates firmly together. In machine riveting, the die is a part of the hammer which is operated by air, hydraulic or steam pressure.

3.1.3 Types of Riveted Joints

Following are the two types of riveted joints, depending upon the way in which the plates are connected.

a) Lap Joint

A lap joint is that in which one plate overlaps the other and the two plates are then riveted together.

b) Butt Joint

A butt joint is that in which the main plates are kept in alignment butting (i.e. touching) each other and a cover plate (i.e. strap) is placed either on one side or on both sides of the main plates. The cover plate is then riveted together with the main plates. Butt joints are of the following two types :

1. Single strap butt joint, and 2. Double strap butt joint.

In a single strap butt joint, the edges of the main plates butt against each other and only one cover plate is placed on

one side of the main plates and then riveted together.

In a double strap butt joint, the edges of the main plates

butt against each other and two cover plates are placed on both sides of the main plates and then riveted together.

In addition to the above, following are the types of riveted joints depending upon the number of rows of the rivets.

- Single riveted joint, and 2. Double riveted joint.

3.1.4 Failures of a Riveted Joint

A riveted joint may fail in the following ways :

1. Tearing of the plate at an edge. A joint may fail due to tearing of the plate at an edge as shown in Fig. 9.13. This can be avoided by keeping the margin, m = 1.5d, where d is the diameter of the rivet hole.

2. Tearing of the plate across a row of rivets. Due to the tensile stresses in the main plates, the main plate or cover plates may tear off across a row of rivets as shown in Fig. 9.14. In such cases, we consider only one pitch length of the plate, since every rivet is responsible for that much length of the plate only. The resistance offered by the plate against tearing is known as tearing resistance or tearing strength or tearing value of the plate.

Let  p= Pitch of the rivets, d = Diameter of the rivet hole, t = Thickness of the plate, and

t = Permissible tensile stress for the plate material.

We know that tearing area per pitch length,

A t = (p – d )t

∴Tearing resistance or pull required to tear off the plate per pitch length,

Pt= At.t = (p – d) t.t

When the tearing resistance (Pt) is greater than the applied load (P) per pitch length, then this type of failure will not occur.

3. Shearing of the rivets. The plates which are connected by the rivets exert tensile stress on the rivets, and if the rivets are unable to resist the stress, they are sheared off as shown in Fig. 9.15. It may be noted that the rivets are in *single shear in a lap joint and in a single cover butt joint, as shown in Fig. 9.15. But the rivets are in double shear in a double cover butt joint as shown in Fig. 9.16. The resistance offered by a rivet to be sheared off is known as shearing resistance or shearing strength or shearing value of the rivet.

4. Crushing of the plate or rivets. Sometimes, the rivets do not actually shear off under the tensile stress, but are crushed as shown in Fig. 9.17. Due to this, the rivet hole becomes of an oval shape and hence the joint becomes loose. The failure of rivets in such a manner is also known as bearing failure. The area which resists this action is the projected area of the hole or rivet on diametral plane .The resistance offered by a rivet to be crushed is known as crushing resistance or crushing strength or bearing value of the rivet.

Let d = Diameter of the rivet hole, t = Thickness of the plate,c= Safe permissible crushing stress for the rivet or

plate material, and n = Number of rivets per pitch length under crushing.

We know that crushing area per rivet (i.e. projected area per rivet),

Ac = d.t

Total crushing area = n.d.t

and crushing resistance or pull required to crush the rivet per pitch length,

Pc = n.d.t.c

When the crushing resistance (Pc) is greater than the applied load (P) per pitch length, then this type of failure will occur.

WELDED JOINTS

3.2.1 Introduction

A welded joint is a permanent joint which is obtained by the fusion of the edges of the two parts to be joined together, with or without the application of pressure and a filler material. The heat required for the fusion of the material may be obtained by burning of gas (in case of gas welding) or by an electric arc (in case of electric arc welding). The latter method is extensively used because of greater speed of welding.

3.2.2 Welding Processes

The welding processes may be broadly classified into the following two groups:

1. Welding processes that use heat alone e.g. fusion welding.

2. Welding processes that use a combination of heat and pressure e.g. forge welding

3.2.3 Types of Welded Joints

Following two types of welded joints are important from the subject point of view:

- Lap joint or fillet joint, and 2. Butt joint

a) Lap Joint

The lap joint or the fillet joint is obtained by overlapping the plates and then welding the edges of the plates. The cross-section of the fillet is approximately triangular. The fillet joints may be

1. Single transverse fillet, 2. Double transverse fillet, and 3. Parallel fillet joints.

The fillet joints are shown in Fig. 10.2. A single transverse fillet joint has the disadvantage that the edge of the plate which is not welded can buckle or warp out of shape.

b) Butt Joint

The butt joint is obtained by placing the plates edge to edge as shown in Fig. 10.3. In butt welds,

the plate edges do not require bevelling if the thickness of plate is less than 5 mm. On the other hand, if

the plate thickness is 5 mm to 12.5 mm, the edges should be bevelled to V or U-groove on both sides.

3.2.4 Basic Weld Symbols

The basic weld symbols according to IS : 813 – 1961 (Reaffirmed 1991) are shown in the following table.

.

3.2.5 Strength of Transverse Fillet Welded Joints

We have already discussed that the fillet or lap joint is obtained by overlapping the plates and

then welding the edges of the plates. The transverse fillet welds are designed for tensile strength. Let

us consider a single and double transverse fillet welds as shown in Fig. 10.6 (a) and (b) respectively.

In order to determine the strength of the fillet joint, it is assumed that the section of fillet is a right angled triangle ABC with hypotenuse AC making equal angles with other two sides AB and BC. The enlarged view of the fillet is shown in Fig. 10.7. The length of each side is known as leg or size of the weld and the perpendicular distance of the hypotenuse from the intersection of legs (i.e. BD) is known as throat thickness. The minimum area of the weld is obtained at the throat BD, which is given by the product of the throat thickness and length of weld.3.2.6 Special Cases of Fillet Welded Joints

The following cases of fillet welded joints are important from the subject point of view.

This shear stress occurs in a horizontal plane along a leg of the fillet weld. The maximum shear occurs on the throat of weld which is inclined at 45° to the horizontal plane. 							Length of throat, t = s sin 45° = 0.707 s

3.2.7 Stress Concentration Factor for Welded Joints

The reinforcement provided to the weld produces stress concentration at the junction of the weld and the parent metal. When the parts are subjected to fatigue loading, the stress concentration factor as given in the following table should be taken into account.

BOLTED JOINTS