Course outline :            Turbo-Machinery  (MEng3201)

| Department of Mechanical Engineering/Institute of Technology Debre Markos University |  |
| --- | --- |
| Course Title | Turbo-Machinery |
| Course Code | MEng3201 |
| Degree Program | BSc in Mechanical Engineering |
| Module Name | Energy Conversion Machines |
| Course Instructor | HARISH H V |
| ECTS Credits | 5 |
| Contact Hours (per week) | 5 |
| Lecture/Contact | (32 lecture +48 tutorial) |
| Year/Semester | 5th |
| Prerequisite | Fluid Mechanics, Thermodynamics II |
| Status of the course | Compulsory |
| Course outline | 1. Introduction 1.1. Introduction 1.2. Classification of Turbomachinery 1.3. Application 1.4. Thermodynamics 1.4.1. Basic thermodynamics 1.4.2. Adiabatic flow through nozzles 1.4.3. Adiabatic flow through diffusers 1.5. Compressible flow 1.6. Basic relations 2. Centrifugal pumps and fans 2.1. Introduction 2.2. Impeller flow 2.3. Efficiency 2.4. Performance characteristics 2.5. Design of pumps 2.6. Fans 3. Centrifugal compressors 3.1. Introduction 3.2. Impeller design 3.3. Diffuser design 3.4. Performance |
|  | 4. Axial-flow pumps and fans 4.1. Introduction 4.2. Stage pressure rise 4.3. Losses 4.4. Pump design 4.5. Fan design 5 Pump selection guidelines, Cavitation and water hammer problems in pump system 6 Axial-flow compressor 6.1. Introduction 6.2. Basic theory 6.3. Cascade tests 6.4. Performance 7 Gas turbines 7.1. Introduction 7.2. Basic theory 7.3. Design 7.4. Radial-flow turbines 8 Steam turbines 8.1. Introduction 8.2. Impulse turbines 8.3. Reaction turbines 9. Hydraulic turbines 9.1. Introduction 9.2. Pelton wheel 9.3. Francis turbine 9.4. Kaplan turbine 9.5. Cavitation |
| Teaching & Learning Methods | Lectures (32hrs)  Tutorials on lectures (48hrs)  Home study including Project, Field Visit, Personal study and assignments (55 hrs) |
| Evaluation systems | Assignments: 10% Quiz exam: 10% Mid exam: 30% (continuous assessment) Final Exam: 50% |
| Course Policy | Minimum attendance required to be permitted to examination:80% 100% attendance during laboratory sessions |
| References | 1. Earl Logan, ―Turbomachinery, basic theory and application‖, Marcel dekker, New york and basel 2. F. M. White, "Fluid Mechanics", 3rd, 4th or 5th Ed., McGraw-Hill 1994 3. Cohen & Rogers, ―Gas turbine theory and practice‖ 4. W. J. Keartin, ―Steam Turbine theory and practice‖ 5. Karassik, Pump Handbook, McGraw-Hill, 1985 6. S. L. Dixon, Fluid Mechanics Thermodynamics of Turbo-machinery, Pergamon Press, 1994. 7. R. K. Turton, Principles of Turbomachinery, Chapman and Hall, 1995. 8. R. I. Lewis, Turbomachinery Performance Analysis, Arnold, 1996. 9. Fluid mechanics and Thermodynamics of Turbo Machinery – S.L.Dixon, Butterworth Heinemann, Feb 23, 2005 10. Rama S.R. Gorla and Aijaz A. Khan, Turbomachinery: Design and Theory (Mechanical Engineering (Marcell Dekker)), Aug 12, 2003. 11. Logan, Handbook of Turbo machinery, 2nd 2003 |

Instructor Name

Mr. HARISH H V

Signature: ____________

Date:      ____________

CHAPTER 1

INTRODUCTION AND CLASSIFICATION OF TURBOMACHINES

TURBOMACHINE

ADIABATIC FLOW THROUGH NOZZLE

Let m1 is the mass entering the steady flow device at velocity V1  and pressure p1 and m2 is the mass leaving the steady flow device at outlet with a velocity of V2 and pressure p2.

According to steady flow energy equation

h1+V12/2 +z1g  +q   = h2 +V22/2 +z2g +w--------equ (2.1)

where suffix 1 indicates inlet and 2 indicates outlet. Since nozzle is an adiabatic device and it is an energy transformation device.

Q=0,w=0,neglecting potential energy

Equation (2.1) becomes

h1+V12/2 = h2 +V22/2

Since inlet entry condition of nozzle is stagnated; that is flow is adiabatically brought to zero  V1=0

h1 = h2 +V22/2

V2 = 2h1-h2

For gases V2 = 2T1-T2

COMPRESSIBLE FLOW AND BASIC RELATION

CHAPTER 2

CENTRIFUGAL PUMPS AND FANS

Classification and parts of centrifugal pump:

Rotodynamic Pumps

A roto dynamic pump is a device where mechanical energy is transferred from the rotor to the fluid by the principle of fluid motion through it. The energy of the fluid can be sensed from the pressure and velocity of the fluid at the delivery end of the pump. Therefore,it is essentially a turbine in reverse. Like turbines, pumps are classified according to the main direction of fluid path through them like (i) radial flow or centrifugal, (ii) axial flow and (iii)mixed flow types.

Centrifugal Pumps

The pumps employing centrifugal effects for increasing fluid pressure have been in use for

more than a century. The centrifugal pump, by its principle, is converse of the Francis

turbine. The flow is radically outward, and the hence the fluid gains in centrifugal head while

flowing through it. Because of certain inherent advantages, such as compactness, smooth and

uniform flow, low initial cost and high efficiency even at low heads, centrifugal pumps are

used in almost all pumping systems. However, before considering the operation of a pump in

detail, a general pumping system is discussed as follows.

CHAPTER3

CENTRIFUGAL COMPRESSOR

CHAPTER4

AXIAL FLOW PUMP

CHAPTER 5

Pump selection guidelines, cavitation, priming

CHAPTER6

AXIAL FLOW COMPRESSOR

CHAPTER7

GAS TURBINE

VELOCITY TRIANGLE OF GAS TURBINE

CHAPTER8

STEAM TURBINE

CHAPTER9

HYDRAULIC TURBINE